(Sample program to drill six holes on the Microfluidics Lab Drill)

(Dave Bothman, 2 July 2012)

(Change x,y coordinates of G73 commands for your hole coordinates)

(Setup blocks)

G21 (select mm units)

G49 G80 G90 (cancel offsets, cancel cycles, absolute coordinates)

G98 (Move the drill away from the part when drilling is finished)

G00 X0 Y0 Z5 (move to the lower left corner of slide, 5mm above)

(Hole drilling blocks)

G73 X22.5 Y12.5 Z-1.2 Q.1 R0.5 F5

G73 X29.5 Y4.5 Z-1.2 Q.1 R0.5 F5

G73 X45.5 Y4.5 Z-1.2 Q.1 R0.5 F5

G73 X52.5 Y12.5 Z-1.2 Q.1 R0.5 F5

G73 X45.5 Y20.5 Z-1.2 Q.1 R0.5 F5

G73 X29.5 Y20.5 Z-1.2 Q.1 R0.5 F5

(Closing block)

M30